Amennyiben probléma lenne a magas bitrátájú állomások lejátszásával:

Forrás: https://4pda.to/forum/index.php?showtopic=1010378&st=16700#entry132067099

Régóta próbálom kideríteni, hogy miért működnek egyes internetes állomások megszakításokkal és állandó rángatásokkal.

A cél a zajmentes hangzás elérése volt. Például: "Iskatel 320 aac" http://iskatel.hosting…u:8015/iskatel-320.aac

E-rádión ESP32-Wrover + VS1053B / I2S-sel.

Hetekig próbálkoztam ezzel, és megtaláltam a megoldást!

Wroveren, sőt Wroomon is működött interferencia és rángatás nélkül.

Nem a rossz internetről van szó, hanem az Arduino könyvtárakról .

Egy kis háttértörténet:

A yoRadio Arduinón van lefordítva, és az Arduino már lefordított ESP-IDF-et ( https://github.com/espressif/esp-idf ) használ sok könyvtárral .

Az ESP-IDF fordításakor bármit konfigurálhatsz, de az Arduino az alapértelmezett konfigurációt használja, és nincs mód a javításra.

Találtam egy jó megoldást a https://github.com/sch…Better_WiFi_throughput oldalon . Az internettel kapcsolatos probléma az ESP-IDF SDK LWIP könyvtára által használt maximális TCP kapcsolatok számában volt.

Újrafordítottam az ESP-IDF-et új LWIP paraméterekkel:

CONFIG_LWIP_MAX_ACTIVE_TCP=512

CONFIG_LWIP_MAX_LISTENING_TCP=512

CONFIG_LWIP_TCP_SND_BUF_DEFAULT=8192

CONFIG_LWIP_TCP_WND_DEFAULT=32768

CONFIG_LWIP_TCP_RECVMBOX_SIZE=32

CONFIG_LWIP_TCP_QUEUE_OOSEQ=y,

hogy megkapjam az új liblwip.a könyvtárat, és lecseréljem azt az Arduino-ban a régi helyett a 2.0.X kernelhez.

A liblwip cseréjének lépései:

1. Töltsd le a liblwip-fix.zip fájlomat.

2. Csomagold ki és másold át a liblwip.a fájlt az alábbi könyvtárba:

ESP32 modulnál a fájl elérési útvonala:

C:\Users\user\AppData\Local\Arduino15\packages\esp32\hardware\esp32\2.0.17\tools\sdk\esp32\lib\

vagy

ESP32-S3 modulnál a fájl elérési útvonala:

C:\Users\user\AppData\Local\Arduino15\packages\esp32\hardware\esp32\2.0.17\tools\sdk\esp32s3\lib\

értelemszerűen a user a saját felhasználói könyvtárad

(ebben a könyvtárban lennie kellett egy régi liblwip.a fájlnak, amit lecserélünk az új fájllal)

3. Indítsd újra az Arduinót

4. Fordítsd újra az E-radiót, mint korábban, és töltsd be az ESP32-be

Működik a 2.0.6 - 2.0.17 kernelekkel

Ugyanez a javítás a 3.0.X kernelhez: liblwip_3.0.5.zip

ESP32-S3 modulnál a fájl elérési útvonala:

C:\Users\User\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.1-33fbade6\esp32s3\lib\

ESP32 modulnál a fájl elérési útvonala:

C:\Users\User\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.1-33fbade6\esp32\lib\

(az idf-release_v5.1-33fbade6 könyvtár eltérő lehet)

If you have problems playing high-bitrate stations:

Source: https://4pda.to/forum/index.php?showtopic=1010378&st=16700#entry132067099

I've been trying for a long time to figure out why some internet radio stations play with interruptions and constant stuttering.
My goal was to achieve clean, noise-free audio playback. For example:
"Iskatel 320 aac" http://iskatel.hosting…u:8015/iskatel-320.aac
on E-radio using ESP32-Wrover + VS1053B / I2S.

I experimented with this for weeks and finally found the solution!
It worked smoothly, without interference or stuttering, both on Wrover and even on Wroom modules.
The problem is not with a poor internet connection, but with the Arduino libraries.

Here’s a bit of background:

yoRadio is compiled on Arduino, and Arduino itself uses a precompiled version of ESP-IDF (https://github.com/espressif/esp-idf) along with many libraries.
When you compile ESP-IDF yourself, you can configure everything, but Arduino always uses the default configuration and there is no way to change or fix it directly.

I found a good solution on https://github.com/sch…Better_WiFi_throughput.
The issue with the internet connection was caused by the maximum number of TCP connections set by the LWIP library in the ESP-IDF SDK.

So, I recompiled ESP-IDF with new LWIP parameters:

CONFIG_LWIP_MAX_ACTIVE_TCP=512

CONFIG_LWIP_MAX_LISTENING_TCP=512

CONFIG_LWIP_TCP_SND_BUF_DEFAULT=8192

CONFIG_LWIP_TCP_WND_DEFAULT=32768

CONFIG_LWIP_TCP_RECVMBOX_SIZE=32

CONFIG_LWIP_TCP_QUEUE_OOSEQ=y

to get a new liblwip.a library, which I then replaced in Arduino, overwriting the old one for the 2.0.X core.

How to replace the liblwip library:

- Download my liblwip-fix.zip file.

- Unzip it and copy the liblwip.a file to the following folder:

For ESP32 modules:
C:\Users\user\AppData\Local\Arduino15\packages\esp32\hardware\esp32\2.0.17\tools\sdk\esp32\lib\

For ESP32-S3 modules:
C:\Users\user\AppData\Local\Arduino15\packages\esp32\hardware\esp32\2.0.17\tools\sdk\esp32s3\lib\

Obviously, replace "user" with your own Windows username.
(In this directory, there should already be an old liblwip.a file—overwrite it with the new one.)

- Restart Arduino.

- Recompile your E-radio project as usual, and upload it to your ESP32.

This works with Arduino core versions from 2.0.6 up to 2.0.17.

The same fix is available for the 3.0.X core as liblwip_3.0.5.zip:

For ESP32-S3 modules:
C:\Users\User\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.1-33fbade6\esp32s3\lib\

For ESP32 modules:
C:\Users\User\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.1-33fbade6\esp32\lib\

(Note: The "idf-release_v5.1-33fbade6" directory name may vary in your setup.)